nginx multiple 5 websites launch at once -

 NGINX (pronounced "engine x") is an open-source HTTP web server and reverse proxy known for its high performance, stability, and low resource utilization.


Install NGINX -

sudo apt-get update -y

sudo apt-get install nginx

systemctl status nginx

systemctl start nginx

sudo apt get update ubuntu
 sudo apt get upgrade ubuntu

 nginx -v
 systemctl start nginx
 systemctl enable nginx
 systemctl status nginx

 by default run on port 80
 check using public ip of instance.

 cd/var/www/

html folder is main folder for our website.

 cd html/
 ls
 cat index.nginx-debian.html

  cd/home/ubuntu/
 sudo apt install tree -y

sudo apt install bat -y

 cd -
 var/www/html
 batcat index.nginx-debian.html

 cd ..
 ls

html

mkdir site1 site2 site3 site4 site5

 cd site1

ls

vi index.html
 Esc :wq


 cd site2
 ls
 vi index.html
 Esc :wq

 cd site3
 ls
 vi index.html
 Esc :wq

 cd site4
 ls
 vi index.html
 Esc :wq

 cd site5
 ls
 vi index.html
 Esc :wq

 tree

 html - site1-site2-site3-site4-site5


 cd home/ubuntu

cd etc/nginx/

ls

sites-enabled
sites-available

 sites-enabled = nginx link
 sites-available = config

cd sites-available
 ls

default

 cat default

 vi default

 copy this much configuration from these default file.


 server {
 listen 80;
 listen [::]:80;

server_name example.com;

root /var/www/example.com;

index index.html;

location / {
                try_files $uri $uri/ =404;

}

 }

 cd sites-available
 ls
 config-1 default

 vi config-1

 server {
  listen 210;

server_name site1.com www.site1.com;

 root /var/www/site1;

index index.html;

location / {
                try_files $uri $uri/ =404;

}

 }


 cat config-1

 vi config-2


 server {
  listen 220;

server_name site2.com www.site2.com;

 root /var/www/site2;

index index.html;

location / {
                try_files $uri $uri/ =404;

}

 }

 same for next 3

 vi config-3

 server {
  listen 230;

server_name site3.com www.site3.com;

 root /var/www/site3;

index index.html;

location / {
                try_files $uri $uri/ =404;

}

 }

 vi config-4

 server {
  listen 240;

server_name site4.com www.site4.com;

 root /var/www/site4;

index index.html;

location / {
                try_files $uri $uri/ =404;

}

 }

 vi config-5

 server {
  listen 250;

server_name site5.com www.site5.com;

 root /var/www/site5;

index index.html;

location / {
                try_files $uri $uri/ =404;

}

 }


 cd etc/nginx
 ls

cd sites-available
 ls
 config-1 config-2 config-3 config-4 config-5 default

 cd sites-enabled
 ls
 default

ln -s /etc/nginx/sites-available/config-1 /etc/nginx/sites-enabled/

cd sites-enabled/

ls

config01 default

ln -s /etc/nginx/sites-available/config-2 /etc/nginx/sites-enabled/

ln -s /etc/nginx/sites-available/config-3 /etc/nginx/sites-enabled/

ln -s /etc/nginx/sites-available/config-4 /etc/nginx/sites-enabled/

ln -s /etc/nginx/sites-available/config-5 /etc/nginx/sites-enabled/

ls

config-1 config-2  config-3  config-4  config-5 default


 cd ..

 cd ..

 home/

 systemctl restart nginx
 systemctl reload nginx

 go to console -
 config 210-250 port

 for that -

go to security - security group - edit inbound role -
 add -
 Custom TCP - 210-250    Anywhere-IPV4 Save rules.

 systemctl restart nginx
 systemctl reload nginx

 how to fetch/launch 5 sites -

 take public-ip address - 3.86.2.107
 default website will launch

http://98.81.79.236:210/
http://98.81.79.236:220/

http://98.81.79.236:230/

http://98.81.79.236:240/

http://98.81.79.236:250/

Note - don’t use  110 port number as it is used for post office protocol.